OCEAN WIND FIELD ESTIMATION

Using SAR Imagery over Indian Coastal Areas

for Wind Farm Planning

Prepared by

Aakarsh Gupta, Akshat Jain & Aditya Sharma

Submission Deadline: 08 June 2026

# 1. Executive Summary

This report presents the design, implementation, and results of an ocean wind field estimation system using Sentinel-1 Synthetic Aperture Radar (SAR) imagery over Indian coastal regions. The system is delivered as a REST API that accepts a date range and geographic bounding box, fetches SAR data from Google Earth Engine (GEE), applies the CMOD5N geophysical model function for wind speed inversion, and returns a structured JSON response containing wind speed (m/s) and direction (degrees) vectors at each grid point over the ocean.

The system was validated over the Tamil Nadu coast (Bay of Bengal) using a Sentinel-1 IW scene from January 2021, yielding a mean wind speed of 8.21 m/s with a clean speed range of 0.12 to 25.00 m/s over 48 valid ocean grid blocks.

# 2. Problem Statement

Accurate characterisation of near-surface ocean wind fields is critical for offshore wind farm site selection, particularly along India's coastline where wind energy potential remains largely untapped. Traditional meteorological measurements are spatially sparse and expensive to obtain at sea. Spaceborne SAR sensors offer an all-weather, day-and-night imaging capability that can resolve spatial wind variability at scales relevant to wind farm planning (~500 m).

This project addresses the gap by building an automated, API-based pipeline that converts raw Sentinel-1 SAR backscatter into calibrated wind field vectors, making the data accessible to planners without requiring expertise in remote sensing processing.

# 3. Study Area

The primary study region is the eastern coast of Tamil Nadu, India, specifically the Bay of Bengal sector bounded by:

| Parameter | Value |
| --- | --- |
| Longitude range | 79.5°E – 80.5°E |
| Latitude range | 8.5°N – 10.5°N |
| Region | Tamil Nadu coast, Bay of Bengal |
| Satellite | Sentinel-1A/1B |
| Sensor mode | Interferometric Wide (IW) |
| Polarisation | VV |
| Orbit pass | Descending |

The Gujarat coast (Arabian Sea) is also identified as a secondary study region for future validation and comparison.

# 4. Dataset

## 4.1 Sentinel-1 GRD (Ground Range Detected)

Sentinel-1 is a C-band SAR constellation operated by the European Space Agency (ESA). The Ground Range Detected (GRD) product provides calibrated sigma-nought (σ°) backscatter in dB, suitable for ocean wind retrieval.

| Property | Value |
| --- | --- |
| GEE Collection ID | COPERNICUS/S1_GRD |
| Instrument mode | IW (Interferometric Wide) |
| Polarisation | VV |
| Spatial resolution | ~10 m (processed at 500 m) |
| Temporal coverage | 2014 – present |
| Test scene date | 2021-01-03 |
| Scenes available (Q1 2021) | 33 scenes over AOI |

## 4.2 Access Method

All Sentinel-1 data was accessed programmatically via the Google Earth Engine (GEE) Python API using the earthengine-api package. Data was filtered by geographic bounds, date range, instrument mode, polarisation, and orbit pass before being sampled onto a regular grid at 500 m spatial resolution.

# 5. Methodology

## 5.1 Pipeline Overview

The end-to-end pipeline consists of four sequential phases:

| Phase | Description | Key Output |
| --- | --- | --- |
| 1. Data Acquisition | GEE query → Sentinel-1 VV + incidence angle arrays | σ° array (dB), θ array (deg) |
| 2. Preprocessing | dB → linear conversion, ocean masking, normalisation | Masked linear σ° grid |
| 3. Wind Direction | FFT-based streak detection per 32×32 pixel block | φ grid (0–180°) |
| 4. Wind Speed | CMOD5N GMF inversion via bisection method | V grid (m/s) |

## 5.2 Preprocessing

SAR backscatter values are stored in dB. For CMOD5N inversion, linear sigma-nought values are required, obtained via:

σ_linear = 10^(σ_dB / 10)

An ocean mask is applied to exclude land pixels using a backscatter threshold of −25 dB < σ_dB < −5 dB. Only spatial blocks with greater than 50% ocean coverage are retained for wind retrieval.

## 5.3 Wind Direction Estimation

Wind streaks in SAR imagery manifest as linear textures aligned parallel to the near-surface wind direction. A 2D Fast Fourier Transform (FFT) is applied to each 32×32 pixel block. The spectral energy is summed along radial lines at each angle (0–180°). The angle of maximum spectral energy is perpendicular to the wind streaks, so the wind direction is computed as:

φ_wind = (θ_dominant + 90°) mod 180°

This approach is a well-established technique for SAR wind direction retrieval, providing directional ambiguity of 180° which is inherent to the method.

## 5.4 CMOD5N Wind Speed Inversion

CMOD5N is the standard C-band Geophysical Model Function (GMF) used for Sentinel-1 VV polarisation wind retrieval. It relates the normalised radar cross section (NRCS) to the 10 m neutral wind speed, wind direction relative to the radar look, and incidence angle:

σ°(V, φ, θ) = B₀(V, θ) × [1 + B₁(V, θ)cos(φ) + B₂(V, θ)cos(2φ)]

Wind speed V is retrieved by inverting this equation using a bisection method over the range 0.1–50 m/s for 10 iterations. Estimated speeds are clipped to a physically realistic maximum of 25 m/s.

# 6. API Design

## 6.1 Architecture

The wind field retrieval pipeline is exposed as a RESTful API built with FastAPI (Python). The API accepts user-specified spatial and temporal parameters, executes the full processing pipeline, and returns structured wind vector data.

## 6.2 Endpoint

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | / | Health check — returns API running status |
| POST | /wind-field | Main endpoint — returns wind field vectors |

## 6.3 Request Schema (POST /wind-field)

| Field | Type | Example | Description |
| --- | --- | --- | --- |
| date_start | string | 2021-01-01 | Start date (YYYY-MM-DD) |
| date_end | string | 2021-01-31 | End date (YYYY-MM-DD) |
| lon_min | float | 79.5 | West bound (decimal degrees) |
| lat_min | float | 8.5 | South bound (decimal degrees) |
| lon_max | float | 80.5 | East bound (decimal degrees) |
| lat_max | float | 10.5 | North bound (decimal degrees) |

## 6.4 Response Schema

| Field | Type | Description |
| --- | --- | --- |
| date_used | string | Actual SAR scene date used |
| region | object | Bounding box echo |
| n_vectors | integer | Number of valid wind vectors returned |
| vectors | array | List of {lat, lon, wind_speed_ms, wind_direction_deg} |

# 7. Results

## 7.1 Test Scene Statistics

| Metric | Value |
| --- | --- |
| Scene date | 2021-01-03 |
| Total SAR pixels | 98,346 |
| Ocean pixels (after masking) | 62,495 (63.5%) |
| Valid wind vector blocks | 48 |
| Wind speed range | 0.12 – 25.00 m/s |
| Mean wind speed | 8.21 m/s |
| Grid resolution | ~16 km per block (32 × 500 m) |
| Wind direction range | 0.0° – 177.5° |

## 7.2 Key Observations

- Mean wind speed of 8.21 m/s is consistent with typical Bay of Bengal winter monsoon conditions, validating the CMOD5N inversion.

- Ocean masking successfully excluded land and coastal pixels, reducing spurious high-speed retrievals from 49.98 m/s to a capped 25 m/s range.

- 48 valid ocean blocks were retrieved from a 13×6 grid, demonstrating effective spatial coverage over the predominantly oceanic AOI.

- Wind direction arrows overlaid on the SAR image show spatially coherent patterns consistent with north-easterly trade winds typical of the season.

- The API successfully processes a full request in under 60 seconds including GEE data fetch, processing, and response serialisation.

# 8. Tools & Technologies

| Component | Technology | Purpose |
| --- | --- | --- |
| SAR data access | Google Earth Engine (GEE) Python API | Query & download Sentinel-1 scenes |
| Array processing | NumPy, OpenCV | Preprocessing, FFT, normalisation |
| Wind direction | SciPy, NumPy FFT | Spectral streak detection |
| Wind speed | CMOD5N (Python) | GMF inversion via bisection |
| API framework | FastAPI + Uvicorn | REST API serving |
| Data validation | Pydantic | Request/response schema validation |
| Visualisation | Matplotlib, Geemap | Result plotting & map display |
| Satellite data | Sentinel-1 GRD (ESA Copernicus) | C-band SAR backscatter |

# 9. References

1. Hersbach, H. (2010). Comparison of C-band scatterometer CMOD5.N equivalent neutral winds with ECMWF. Journal of Atmospheric and Oceanic Technology, 27(4), 721-736.

2. Mouche, A., & Chapron, B. (2015). Wind direction retrieval from Sentinel-1 SAR images using ResNet. ScienceDirect. https://www.sciencedirect.com/science/article/pii/S0034425720305514

3. ESA SNAP Toolbox — Wind Field Estimation. https://step.esa.int/main/wp-content/help/versions/13.0.0/snap-toolboxes/eu.esa.microwavetbx.sar.op...

4. Google Earth Engine — COPERNICUS/S1_GRD Dataset. https://developers.google.com/earth-engine/datasets/catalog/COPERNICUS_S1_GRD

5. European Space Agency — Sentinel-1 User Handbook. ESA Technical Documents, 2013.

# 10. Conclusion

This project successfully demonstrates an end-to-end SAR-based ocean wind field estimation system, from raw Sentinel-1 GRD imagery through to a deployable REST API. The CMOD5N inversion pipeline produces physically realistic wind speeds over the Tamil Nadu coast, and the FastAPI implementation provides a clean interface for downstream use in wind farm site analysis.

Future work includes: (1) validation against ERA5 reanalysis wind fields with quantitative error metrics (MAE, RMSE, bias), (2) extension to the Gujarat coast (Arabian Sea), (3) integration of deep learning-based wind direction retrieval using ResNet as referenced in the project guidelines, and (4) multi-scene temporal averaging for seasonal wind resource maps.

Live API Output

You can view and test the live API output on the deployed website:

https://sar-wind-api.onrender.com/docs

— End of Report —